אוניברסיטת בן-גוריון בנגב

Ben-Gurion University of the Negev

הפקולטה למדעי ההנדסה

בית הספר להנדסת חשמל ומחשבים

המחלקה להנדסת חשמל ומחשבים

Faculty of Engineering Science

School of Electrical and Computer Engineering

Dept. of Electrical and Computer Engineering

פרויקט הנדסי שנה ד׳

Fourth Year Engineering Project

PDR

בי-סנס

BeeSense

| Project number: | p-2027-xxx |
| --- | --- |
| Students (name & ID): | Student 1 name – ID Student 2 name – ID |
| Supervisor: | Supervisor name |
| Sponsors: | — |
| Submitting date: | 25.10.2026 |

בי-סנס

שמות הסטודנטים: שם סטודנט/ית 1, שם סטודנט/ית 2

user1@post.bgu.ac.il

user2@post.bgu.ac.il

שם המנחה: שם המנחה

תקציר

דבורי הדבש חיוניות להאבקת גידולים חקלאיים, אך מצב הכוורת נבדק כיום בעיקר בפתיחה ידנית ותקופתית שלה, ובין ביקור לביקור המגדל אינו יודע מה מתרחש בכוורת. בריחת נחיל, למשל, גורמת לאובדן של חלק ניכר מאוכלוסיית המושבה, ולעיתים קרובות המגדל מגלה זאת רק בביקור הבא. בי-סנס היא מערכת IoT לניטור רציף של כוורות דבורים. בכל כוורת מותקן צומת חיישנים מבוסס מיקרו-בקר הפועל על סוללה: חיישן תאוצה (רטט) המוצמד למסגרת הוולדות, ארבעה תאי עומס מתחת לכוורת, וחיישני טמפרטורה ולחות. הצומת מחשב בעצמו את ספקטרום תדרי הזמזום בתחום 50 עד 1000 הרץ ואת משקל הכוורת, ושולח תמצית קטנה של הנתונים בתקשורת LoRa לשער (Gateway) מרכזי בכוורן. בשער פועלים אלגוריתמים לזיהוי בריחת נחיל ולזיהוי חריגות המעידות על מצוקת המושבה, והנתונים וההתרעות מוצגים למגדל בממשק Web. החדשנות היא שילוב של ניתוח רטט ספקטרלי עם מדידת משקל בצומת סוללתי בצריכת הספק נמוכה, לעומת מערכות מחקר הצורכות כמה וואטים מרשת החשמל, ומוצרים מסחריים המודדים משקל בלבד. נבחן את המערכת בכוורן פעיל לאורך עונת הנחילה באביב, ונמדוד את דיוק השקילה (יעד: 20± גרם), אחוז קליטת החבילות (יעד: מעל 95%), צריכת הזרם הממוצעת (יעד: מתחת למיליאמפר אחד, כלומר לפחות חצי שנה על סוללה), ודיוק (Precision) ורגישות (Recall) בזיהוי אירועי נחילה ואירועים מבוקרים (יעד: מעל 90%). חיזוי מוקדם של נחילה ואבחון מחלות ספציפיות הם כיווני מחקר נלווים, ולא תוצרים מחייבים של פרויקט זה.

מילות מפתח: האינטרנט של הדברים, ניטור כוורות, דבורת הדבש, חיישני רטט, עיבוד אותות, צפיפות הספק ספקטרלית, תאי עומס, מערכות משובצות בהספק נמוך, LoRa, זיהוי חריגות.

BeeSense

Students Names: Surname1 Name1, Surname2 Name2

user1@post.bgu.ac.il

user2@post.bgu.ac.il

Adviser Name: Surname Name

Abstract

Honeybees are essential crop pollinators, yet colonies are still checked mainly by opening the hive every week or two, and between visits the beekeeper does not know what happens inside. A swarm, for example, takes a large part of the colony away, and is often discovered only at the next visit. BeeSense is an Internet of Things (IoT) system for continuous hive monitoring. Each hive carries a battery-powered microcontroller node with an accelerometer (vibration sensor) attached to the brood frame, four load cells under the hive, and temperature and humidity sensors. The node computes the buzzing spectrum between 50 and 1000 Hz and the hive weight on board, and sends a compact summary over LoRa to a gateway at the apiary. The gateway runs algorithms that detect swarm departures and anomalies that indicate colony distress, and a web dashboard shows the data and alerts to the beekeeper. The innovation is the fusion of spectral vibration analysis and weight in a low-power battery node, compared with research systems that draw several watts from mains power and commercial products that measure weight only. We will evaluate the system at a working apiary through the spring swarming season and measure weighing accuracy (target: ±20 g), packet delivery ratio (target: above 95%), average current (target: below 1 mA, i.e. at least six months per charge), and event-level precision and recall for swarms and controlled events (target: above 90%). Early swarm prediction and diagnosis of specific diseases are exploratory directions, not committed outcomes of this project.

Keywords: Internet of Things, Hive Monitoring, Honeybees, Vibration Sensing, Signal Processing, Power Spectral Density, Load Cells, Low-Power Embedded Systems, LoRa, Anomaly Detection.

1. Spec Sheet

BeeSense – low-power vibration and weight monitoring of honeybee hives

1.1 Motivation, Use Case and Applications

Honeybee colonies are inspected mostly by opening the hive every one to two weeks; inspections disturb the colony, cost labour, and leave the beekeeper blind between visits. Precision beekeeping, i.e. monitoring colonies with sensors so that interventions are based on data, was proposed to close this gap [1]. Hive vibrations change measurably before swarming [2], [3], and continuous hive weight reveals foraging, consumption and swarm departures [4]. Initial use case: a beekeeper with tens of Langstroth hives at a remote apiary without mains power, visiting weekly. The system must alert when a swarm leaves or a colony behaves abnormally, and show which hives need a visit first. Applications: commercial and hobby apiaries, pollination services, and research apiaries that need long continuous records.

1.2 Objectives and Work Stages

Objectives:

(1) a battery-powered hive node that measures the vibration spectrum, weight and temperature and runs at least six months per charge (targets in Section 3.3);

(2) a reliable LoRa link from several hives to a gateway, with a database and a web dashboard;

(3) detection of swarm departures and colony anomalies with alerts, and an exploratory study of swarm-preparation indicators.

Stages:

(1) define requirements with a cooperating beekeeper; bench-test the vibration chain (known tones) and the weighing chain (reference masses).

(2) build the node: acquisition, on-board spectrum, power modes; compare candidate vibration sensors; bring up the LoRa link and gateway.

(3) implement the database, detection algorithms and dashboard; install on hives before the spring swarming season.

(4) field evaluation and analysis.

1.3 Technology, Interfaces and Contribution

Each hive node uses a low-power microcontroller (MCU) with a floating-point unit (candidate: ESP32-S3) and a sub-GHz LoRa transceiver (SX1262 class; frequency plan per Israeli regulation). Vibration is measured by a low-noise MEMS accelerometer attached to the central brood frame, as in [3] (candidate: ADXL355, output rate up to 4 kHz); a MEMS microphone and a piezo disc are benchmarked as alternatives in Stage 2. Weight: four 50 kg load cells under a hive platform, read by an HX711 24-bit bridge ADC. In-hive temperature (DS18B20) and outside temperature and humidity (BME280). Power: two 18650 Li-ion cells and an optional small solar panel. Interfaces: SPI (accelerometer), two-wire serial (HX711), 1-Wire and I²C (environment), LoRa to a Raspberry Pi gateway, Wi-Fi or cellular backhaul, and a dashboard in a web browser. Our contribution is a battery-powered node that reduces raw vibration to spectral features inside the hive and fuses them with weight to detect colony events, with measured accuracy, link reliability and energy.

Fig. 1.  BeeSense block diagram (blue: BeeSense system, grey: external).

2. Existing Solutions and Prior Work

Ferrari et al. recorded hive sounds with in-hive microphones and found that the power spectral density near 110 Hz rises, and shifts upward in frequency, before swarming [2]. Ramsey et al. placed accelerometers in the heart of the colony and, by tracking how the vibrational spectra evolve, separated colonies preparing to swarm with more than 90% accuracy, up to 30 days ahead [3]. Cecchi et al. built a multi-sensor hive with microphones, weighing to ±10 g and CO₂, but each hive used a Raspberry Pi drawing about 4 W from mains power [5]. Commercially, BroodMinder hive scales log weight only [6], and BeeHero in-hive sensors measure acoustics and climate and report over Bluetooth Low Energy to a cellular gateway in a closed system [7] (Table I). We chose a frame-mounted accelerometer over an air microphone because it couples directly to the comb and is less sensitive to wind and outside noise.

TABLE I.  PRIOR WORK AND WHAT BEESENSE MUST DEMONSTRATE

| Comparator | Relevant capability | What BeeSense must show |
| --- | --- | --- |
| Ferrari [2], Ramsey [3] | In-hive sound/vibration features precede swarming | Same features on a low-power node, in the field |
| Cecchi et al. [5] | Microphones, weight ±10 g, CO₂; mains, ~4 W per hive | Battery operation, ~mW average power |
| BroodMinder [6] | Hive scale, 5 g resolution, hourly, AA batteries | Added vibration channel and event alerts |
| BeeHero [7] | In-hive acoustics and climate, BLE + cellular | Open, measured pipeline; weight fusion |

3. Proposed Solution

3.1 Signal Processing and Event Detection

Every reporting interval Tc (default 10 min) the node wakes, records Ta = 10 s of vibration at fs = 4 kHz, and averages the weight over 1 s at 10 samples/s. The power spectral density S(f) is estimated with Welch's method (4096-point Hann windows, 50% overlap, Δf = fs/4096 ≈ 0.98 Hz) and reduced to K = 24 logarithmically spaced bands Bk inside the band B = 50–1000 Hz, which covers the swarming band reported in [2]. Relative band energies cancel the sensor coupling and gain:

ek = Σf∈Bk S(f) / Σf∈B S(f),     k = 1, …, K	(1)

z = (x − μh) / σh,     ΔW(t) = W(t) − W(t − τ)	(2)

Each ≈50-byte payload (24 band energies, total vibration power, weight, temperatures, battery voltage) is sent over LoRa. Since colony activity follows a daily cycle [4], the gateway compares each feature x with the same hive's mean μh and deviation σh at the same hour of day over the previous 7 days; an anomaly is declared when |z| > 3 for three consecutive reports. A swarm departure is declared when ΔW < −Wth within τ = 30 min in daytime, together with a vibration-power anomaly. A prime swarm carries roughly 10,000–20,000 bees, about 1–2 kg at ≈0.1 g per bee, so Wth = 0.8 kg is an initial value, tuned on data (thresholds are initial values). Rain, feeding and inspections also change weight: inspections are logged in the dashboard and excluded, and an empty-hive control platform separates environmental drift [4]. On an anomaly the node also sends the full-resolution spectrum (≈1 kB, several packets). Swarm-preparation indicators, following the spectral-trend approach of [3], are studied offline against inspection records.

3.2 Feasibility – Data, Link, Weighing and Power Budgets

There is no prototype yet, so feasibility is shown by analysis from datasheet values; each number below will be replaced by a bench measurement in Stage 2. Data: raw vibration is 4000 samples/s × 2 bytes = 8 kB/s (≈690 MB per hive per day), far beyond a LoRa link; on-board features reduce it to ≈50 bytes per 10 min (≈7 kB/day). Link: at spreading factor SF9, 125 kHz bandwidth and coding rate 4/5, a 50-byte packet takes ≈0.39 s of air time [8], 0.07% of a 10-min interval; even SF12 (≈2.3 s) stays below 0.4%. Weighing: four 50 kg cells at 1 mV/V and 3.3 V excitation give ≈3.3 mV at the 200 kg full scale, i.e. 16.5 µV/kg; the HX711 input noise of 50 nV rms at 10 samples/s [9] equals ≈3 g, well below the 20 g target, so load-cell temperature drift, not ADC noise, will set field accuracy. Power: for task currents Ii lasting ti in a cycle of length Tc,

Ī = Σi Iiti / Tc,     Tlife = η C / Ī	(3)

With estimates of 40 mA for the MCU during the 10.4 s of acquisition and FFT, 120 mA for 0.39 s of LoRa transmission, 1.5 mA for the HX711, 0.2 mA for the accelerometer and 20 µA in deep sleep, Ī ≈ 0.8 mA at Tc = 10 min; with two 3000 mAh cells in parallel and usable fraction η = 0.8, Tlife ≈ 250 days (Fig. 2). The MCU active time is ≈87% of the charge, so the record length Ta and the FFT implementation are the main design levers. Summer heat under the hive lid can exceed Li-ion charging limits, so the enclosure is shaded and charging is cut off by temperature.

Fig. 2.  Estimated battery life vs. reporting interval by (3), for three vibration record lengths (datasheet-level estimates, to be measured).

3.3 Evaluation Plan

Lab: weighing linearity and repeatability with reference masses from 0 to 100 kg and a temperature sweep; the vibration chain is checked with known tones (100–1000 Hz) from a speaker or shaker coupled to a frame; average current is measured with a power analyser over full cycles. Field: 3–5 instrumented hives and one empty control platform at a cooperating apiary name, from installation through the spring swarming season. Ground truth: weekly inspection records (queen cells, brood, queen presence), beekeeper reports and, where possible, an entrance camera to time swarm departures. Natural swarms are rare, so detection is also tested with controlled events, removal of known masses (0.5–2 kg) and short disturbances, scheduled without the detector settings being changed. Each alert matches at most one labelled event within ±30 min, giving precision P = TP/(TP+FP) and recall R = TP/(TP+FN); false alarms are reported per hive-week. Thresholds are tuned on the first weeks and frozen before the evaluation period. The packet delivery ratio (PDR) is computed from sequence numbers at each hive's distance from the gateway.

TABLE II.  PROPOSED ACCEPTANCE TARGETS (FROZEN BEFORE THE FIELD EVALUATION)

| Area | Metric | Target |
| --- | --- | --- |
| Weighing | Error vs. reference masses; daily drift (control) | ≤ ±20 g; reported |
| Vibration chain | Tone frequency error, 100–1000 Hz | ≤ 2 Hz |
| Event detection | Precision / recall (swarms + controlled events) | > 90% / > 90% |
| False alarms | Alerts without a matching event | ≤ 1 per hive-week |
| Wireless | Packet delivery ratio at apiary distance | ≥ 95% |
| Power / reliability | Average current; unattended operation | ≤ 1 mA; ≥ 30 days |

References

[1]	A. Zacepins, V. Brusbardis, J. Meitalovs, and E. Stalidzans, "Challenges in the development of precision beekeeping," Biosyst. Eng., vol. 130, pp. 60–71, Feb. 2015, doi: 10.1016/j.biosystemseng.2014.12.001.

[2]	S. Ferrari, M. Silva, M. Guarino, and D. Berckmans, "Monitoring of swarming sounds in bee hives for early detection of the swarming period," Comput. Electron. Agric., vol. 64, no. 1, pp. 72–77, 2008, doi: 10.1016/j.compag.2008.05.010.

[3]	M.-T. Ramsey, M. Bencsik, M. I. Newton, M. Reyes, M. Pioz, D. Crauser, N. Simon Delso, and Y. Le Conte, "The prediction of swarming in honeybee colonies using vibrational spectra," Sci. Rep., vol. 10, Art. no. 9798, Jun. 2020, doi: 10.1038/s41598-020-66115-5.

[4]	W. G. Meikle and N. Holst, "Application of continuous monitoring of honeybee colonies," Apidologie, vol. 46, no. 1, pp. 10–22, 2015, doi: 10.1007/s13592-014-0298-x.

[5]	S. Cecchi, S. Spinsante, A. Terenzi, and S. Orcioni, "A smart sensor-based measurement system for advanced bee hive monitoring," Sensors, vol. 20, no. 9, Art. no. 2726, May 2020, doi: 10.3390/s20092726.

[6]	BroodMinder, "BroodMinder-W3 four point weight scale kit." [Online]. Available: https://workerandhive.com/products/broodminder-w3-four-point-weight-scale-kit

[7]	BeeHero, "Technology." [Online]. Available: https://www.beehero.io/technology

[8]	Semtech Corp., "LoRa modem designer's guide," Application Note AN1200.13, Jul. 2013.

[9]	Avia Semiconductor, "HX711: 24-bit analog-to-digital converter (ADC) for weigh scales," datasheet.

הערכה לשיחת סקר תכנון ראשוני (PDR)

מספר הפרויקט: P-2027-xxx, שם הפרויקט: בי-סנס – BeeSense

שם המנחה: שם המנחה

שם הסטודנט: שם סטודנט/ית 1          ת.ז.: מספר

| % | קריטריון | 1 - חלש | 2 - בינוני | 3 - מצוין |
| --- | --- | --- | --- | --- |
| 20 | הגדרת המטרה - האם מטרת הפרויקט ברורה? האם הוגדרו קריטריונים ברורים ומדידים כמותית לבחינת הצלחת הפרויקט? |  |  |  |
| 20 | הבנת הבעיה - האם ניכר שהסטודנטים מבינים את הבעיה ואת הפתרונות הקיימים? האם נערך סקר ספרות רלוונטי? |  |  |  |
| 20 | הצעת הפתרון - האם הגישה הכללית ורכיבי הפתרון הוצגו והוסברו בצורה ברורה? האם הובנו מהות העשייה, הצורך וסביבת היישום? |  |  |  |
| 20 | בדיקת היתכנות - האם הסטודנטים ביצעו מחקר בסיסי לבדיקת היתכנות פתרון לבעיה? כמו כתיבת קוד, ניתוח מעגל, ניתוח מודל מתמטי... |  |  |  |
| 20 | איכות כתיבת ה PDR - תחביר, הגדרת המטרה, סקר ספרות, תקצירים, מפרט טכני (או הצעת מחקר) והצעת תכנון מפורטים. |  |  |  |

הערות: ____________________________________________

הערכה לשיחת סקר תכנון ראשוני (PDR)

מספר הפרויקט: P-2027-xxx, שם הפרויקט: בי-סנס – BeeSense

שם המנחה: שם המנחה

שם הסטודנט: שם סטודנט/ית 2          ת.ז.: מספר

| % | קריטריון | 1 - חלש | 2 - בינוני | 3 - מצוין |
| --- | --- | --- | --- | --- |
| 20 | הגדרת המטרה - האם מטרת הפרויקט ברורה? האם הוגדרו קריטריונים ברורים ומדידים כמותית לבחינת הצלחת הפרויקט? |  |  |  |
| 20 | הבנת הבעיה - האם ניכר שהסטודנטים מבינים את הבעיה ואת הפתרונות הקיימים? האם נערך סקר ספרות רלוונטי? |  |  |  |
| 20 | הצעת הפתרון - האם הגישה הכללית ורכיבי הפתרון הוצגו והוסברו בצורה ברורה? האם הובנו מהות העשייה, הצורך וסביבת היישום? |  |  |  |
| 20 | בדיקת היתכנות - האם הסטודנטים ביצעו מחקר בסיסי לבדיקת היתכנות פתרון לבעיה? כמו כתיבת קוד, ניתוח מעגל, ניתוח מודל מתמטי... |  |  |  |
| 20 | איכות כתיבת ה PDR - תחביר, הגדרת המטרה, סקר ספרות, תקצירים, מפרט טכני (או הצעת מחקר) והצעת תכנון מפורטים. |  |  |  |

הערות: ____________________________________________