אוניברסיטת בן-גוריון בנגב

Ben-Gurion University of the Negev

הפקולטה למדעי ההנדסה

בית הספר להנדסת חשמל ומחשבים

המחלקה להנדסת חשמל ומחשבים

Faculty of Engineering Science

School of Electrical and Computer Engineering

Dept. of Electrical and Computer Engineering

פרויקט הנדסי שנה ד׳

Fourth Year Engineering Project

PDR

בי-סנס – ניטור אקוסטי ומשקלי של מושבות דבורי דבש מבוסס האינטרנט של הדברים

BeeSense — IoT-Based Acoustic and Weight Monitoring of Honeybee Colonies

| Project number: | p-2027-165 |
| --- | --- |
| Students (name & ID): | Dror Ben-Simon – 318187747 Dan Cooper – 319011102 |
| Supervisor: | Dr. Dan Vilenchik |
| Sponsors: | — |
| Submitting date: | 25.10.2026 |

בי-סנס – ניטור אקוסטי ומשקלי של מושבות דבורי דבש מבוסס האינטרנט של הדברים

שמות הסטודנטים: דרור בן סימון, דן קופר

drorbe@post.bgu.ac.il

dancoop@post.bgu.ac.il

שם המנחה: ד"ר דן וילנצ'יק

תקציר

מצב כוורות הדבורים נבדק כיום בעיקר בפתיחה ידנית של הכוורת אחת לכמה שבועות עד חודשים, ובין ביקור לביקור המגדל אינו יודע מה מתרחש בה. הכנה לבריחת נחיל נמשכת כשבוע עד שבועיים, והנחיל, שלוקח איתו חלק ניכר מהמושבה, עוזב תוך דקות ולעיתים קרובות מתגלה רק בביקור הבא. בי-סנס היא מערכת מבוססת האינטרנט של הדברים (IoT) לניטור כוורות במכוורת ללא חשמל וללא רשת. בכל כוורת מותקן צומת סוללתי עם חיישן רטט פיאזואלקטרי, המאזין בצריכה של ננו-אמפרים ומעיר את הצומת כשהרטט חורג מרמת הרקע; הצומת מחשב את ספקטרום הרטט והקול ומשלב אותו עם משקל הכוורת והטמפרטורה. מרכזייה סולארית במכוורת אוספת את הנתונים, מזהה סימנים לנחילה מתקרבת וחריגות במושבה, ושולחת התרעות דרך הרשת הסלולרית. החדשנות היא מערכת אוטונומית וזולה שמביאה לשטח יכולת שהוכחה במעבדה, בצריכת הספק נמוכה פי אלפים ממערכות המחקר. היעדים: דיוק שקילה של 20± גרם, קליטת חבילות מעל 95%, לפחות חצי שנה על סוללה, וכיעד המחייב, דיוק (Precision) ורגישות (Recall) מעל 90% בהתרעה על נחילה לפני יציאת הנחיל.

מילות מפתח: האינטרנט של הדברים, ניטור כוורות, דבורת הדבש, חיישן פיאזואלקטרי, השכמה מונעת-אירועים, עיבוד אותות, תאי עומס, מערכות משובצות בהספק נמוך, ESP32, ESP-NOW.

BeeSense — IoT-Based Acoustic and Weight Monitoring of Honeybee Colonies

Students Names: Ben-Simon Dror, Cooper Dan

drorbe@post.bgu.ac.il

Adviser Name: Vilenchik Dan

Abstract

Honeybee colonies are checked mainly by opening the hive every few weeks to months, leaving the beekeeper blind between visits. Swarm preparation takes one to two weeks, and the swarm, taking a large part of the colony, leaves within minutes and is often discovered only at the next visit. BeeSense is an Internet of Things (IoT) system for monitoring hives at apiaries without mains power or network. In each hive, a battery-powered node uses a piezoelectric vibration sensor that listens at nanoampere current and wakes the node when vibration rises above the background; the node then computes vibration and sound spectra and fuses them with hive weight and temperature. A solar-powered hub at the apiary collects the data, detects signs of an approaching swarm and colony anomalies, and sends alerts over the cellular network. The innovation is an autonomous, low-cost system that brings a laboratory-proven capability to the field, at thousands of times less power than research systems. Targets: weighing accuracy of ±20 g, packet delivery above 95%, at least six months per battery and, as the committed target, precision and recall above 90% for swarm alerts issued before the swarm leaves.

Keywords: Internet of Things, Hive Monitoring, Honeybees, Piezoelectric Sensor, Event-Driven Wake-Up, Signal Processing, Load Cells, Low-Power Embedded Systems, ESP32, ESP-NOW.

1. Spec Sheet

BeeSense – event-driven vibration and weight monitoring of honeybee hives

1.1 Motivation, Use Case and Applications

Honeybee colonies are inspected mostly by opening the hive every few weeks to months; inspections disturb the colony, cost labor, and leave the beekeeper blind between visits. Precision beekeeping, i.e. monitoring colonies with sensors so that interventions are based on data, was proposed to close this gap [1]. Swarming is the costliest blind spot: the colony rears new queens for one to two weeks, the prime swarm leaves with the old queen once the queen cells are capped, clusters nearby for a few hours, and is then lost. Hive vibrations change measurably before swarming [2], [3], and continuous hive weight reveals foraging, consumption and swarm departures [4]. Initial use case: a beekeeper with many hives standing close together at a remote apiary without mains power or Wi-Fi, visiting every few weeks. The system must alert before a swarm leaves or a colony behaves abnormally, and show which hives need a visit first. Applications: commercial and hobby apiaries, pollination services, and research apiaries that need long continuous records.

1.2 Objectives and Work Stages

Objectives: (1) a battery-powered hive node that wakes on events, measures vibration, sound, weight and temperature, and lasts at least six months per charge (targets in Section 3.3);

(2) a reliable link from the hives to a solar-powered hub that uploads the data over the cellular network to a web dashboard;

(3) alerts on approaching swarms before the swarm leaves; how early this is possible is the research question.

Stages: (1) requirements with the beekeeper and bench tests of each sensor chain and of the node's energy use; (2) build the node and test the link at the apiary; (3) build the hub, detection and dashboard, and install on hives before spring; (4) field evaluation.

1.3 Technology and Interfaces

Hive node: an ESP32-C3 microcontroller board powered by one 18650 Li-ion cell, with four sensors: (a) a 27 mm piezoelectric disc pressed into the comb at the center of the hive, where comb vibration is strongest (Section 3.2); (b) a small digital MEMS microphone inside the hive, behind a mesh; (c) four 50 kg load cells under the hive with an HX711 weighing ADC; and (d) DS18B20 temperature probes. The piezo disc also feeds a low-power wake circuit (≈2 µA) that wakes the node when vibration jumps above its usual level; an amplifier prepares the signal for the ADC only while recording. The C3 has no floating-point unit, so the spectra are computed with a fixed-point FFT, the usual practice on small microcontrollers. Hub: an ESP32-S3 board at the apiary receives the nodes over ESP-NOW, a direct Wi-Fi radio link that needs no router, uploads over a cellular modem, and runs from an 18 V / 15 W solar panel with a charger and battery. Because the hives stand close together, one hub and one SIM serve a group of hives. Interfaces: ADC (piezo), I²S (microphone), two-wire serial (HX711), 1-Wire (temperature), GPIO (wake-up), ESP-NOW (node to hub), LTE (hub to cloud).

Fig. 1.  BeeSense block diagram (blue: BeeSense system, grey: external; red: event wake-up path).

2. Existing Solutions and Prior Work

Ferrari et al. found with in-hive microphones that the power spectral density near 110 Hz rises, and shifts upward in frequency, before swarming [2]. Ramsey et al. placed accelerometers in the heart of the colony and, by tracking how the vibrational spectra evolve, identified colonies preparing to swarm with more than 90% accuracy, up to 30 days ahead [3]. Cecchi et al. built a multi-sensor hive with microphones, weighing to ±10 g and CO₂, but each hive used a Raspberry Pi drawing about 4 W from mains power [5]. Commercially, BeeHero in-hive sensors measure acoustics and climate and report over Bluetooth Low Energy to a cellular gateway in a closed system [6] (Table I).

TABLE I.  PRIOR WORK AND WHAT BEESENSE MUST DEMONSTRATE

| Comparator | Relevant capability | What BeeSense must show |
| --- | --- | --- |
| Ferrari [2], Ramsey [3] | Sound/vibration features precede swarming | Same features on a battery node, in the field |
| Cecchi et al. [5] | Microphones, weight ±10 g, CO₂; mains, ~4 W per hive | Battery operation, ~1 mW average power |
| BeeHero [6] | In-hive acoustics and climate, BLE + cellular gateway | Open, measured pipeline; weight + vibration fusion |

3. Proposed Solution

3.1 Event-Driven Acquisition, Signal Processing and Detection

The node wakes periodically every Tc = 15 min and on events from the wake circuit; after an event wake it ignores further triggers for 20 min, so a long disturbance costs one acquisition. On every wake it records Ta = 10 s of piezo signal at fs = 2 kHz (ADC with DMA) and of microphone signal (decimated to 2 kHz), and averages the weight over 1 s. For each channel the power spectral density S(f) is estimated with Welch's method (1024-point Hann windows, 50% overlap, Δf ≈ 1.95 Hz) using a fixed-point FFT, since the C3 has no floating-point unit, and reduced to K = 24 logarithmically spaced bands Bk inside B = 50–800 Hz, which covers the swarming band [2] and queen piping (≈200–500 Hz). Relative band energies cancel sensor coupling and gain:

ek = Σf∈Bk S(f) / Σf∈B S(f),     k = 1, …, K	(1)

z = (x − μh) / σh,     ΔW(t) = W(t) − W(t − τ)	(2)

Each ≈75-byte payload (band energies of both channels, total powers, weight, temperatures, battery voltage, wake cause) is sent over ESP-NOW. Since colony activity follows a daily cycle [4], the hub compares each feature x with the same hive's mean μh and deviation σh at the same hour of day over the previous 7 days; an anomaly is declared when |z| > 3 for three consecutive reports. A pre-swarm alert is raised when the band energies in the swarming band (≈100–300 Hz [2]) and the in-hive temperature rise persistently above the hive's own baseline, as reported before swarming in [2]; spectral trends as in [3] are added to extend the warning. A departure is confirmed when ΔW < −Wth within τ = 30 min together with an event wake; a prime swarm carries roughly 10,000–20,000 bees, about 1–2 kg at ≈0.1 g per bee, so Wth = 0.8 kg is an initial value. The time from the first alert to the departure is the measured lead time. Logged inspections are excluded, and an empty-hive control platform separates rain and drift [4].

3.2 Feasibility – Signal Levels, Data, Link and Power Budgets

The parts are in hand but no node is built yet, so feasibility is shown from published measurements and datasheet values, each to be replaced by a bench measurement. Signal levels: comb vibration measured at night with laboratory accelerometers is only 1–5 µm/s² per spectral bin, about ten times weaker on the hive wall [7], and far below the noise of a typical MEMS accelerometer (≈25 µg/√Hz), hence a piezoelectric sensor in the comb. The pre-swarm changes in [2] were found with ordinary in-hive microphones, so our microphone channel is expected to resolve them; whether the piezo chain also resolves the quieter comb vibrations is measured in Stage 1 (fallback: external audio ADC). The departure itself (mass flight, 1–2 kg) gives a robust confirmation. Data and link: raw data is 4 kB/s per channel; features reduce it to ≈75 bytes per report, one ESP-NOW frame (≤250 bytes, 1 Mbps, MAC-level acknowledgement [8]), so 20 hives need ≈5 MB/month of cellular data. Wood, honey and bees attenuate 2.4 GHz, so the radio sits outside the hive body; range is measured on site first (fallback: LoRa on SPI). Weighing: four 50 kg cells at 1 mV/V and 3.3 V excitation give 16.5 µV/kg; the HX711 noise of 50 nV rms at 10 samples/s [9] equals ≈3 g, so load-cell temperature drift will set field accuracy. Node power: for task currents Ii lasting ti per acquisition, A acquisitions per day and a continuous current I0,

Ī = I0 + A Σi Iiti / 86 400 s,     Tlife = η C / Ī	(3)

One acquisition costs ≈0.30 A·s: 22 mA at 80 MHz for the 10 s recording and 28 mA at 160 MHz for ≈1.5 s of FFTs [10], 0.15 s of radio at ≈120 mA average (335 mA TX peak [10]), and the microphone (≈1.4 mA), preamplifier and HX711. With I0 = 22 µA (2 µA wake circuit, 5 µA chip [10], LDO and leakage) and 96 periodic acquisitions per day, Ī ≈ 0.35 mA, and one 3000 mAh cell with η = 0.8 lasts ≈285 days; each extra event wake costs ≈3.4 µA, so the six-month target holds up to ≈60 event wakes per day (Fig. 2), which the adaptive threshold and 20-min hold-off keep well below. Hub power: continuous receive plus the modem draw ≈0.5 W (≈12 Wh/day) against ≈45 Wh/day from the panel (5 full-sun hours, 60% conversion); three 18650 cells cover two days without sun.

Fig. 2.  Estimated node battery life (one 18650 cell) vs. event wake-ups per day by (3) (datasheet-level estimates, to be measured).

3.3 Evaluation Plan

Lab: weighing linearity with reference masses (0–100 kg) and a temperature sweep; piezo and wake chains checked with known tones (100–800 Hz) and bursts from a shaker coupled to a frame; average current measured with a power analyzer over full cycles. Field: 3–5 instrumented hives and one empty control platform at the cooperating apiary name, from installation through the spring swarming season, including a first night-time recording that compares hive signal with the circuit noise floor. Ground truth: weekly inspection records (queen cells, brood, queen presence), beekeeper reports and, where possible, an entrance camera. A swarm is labelled by an observed departure or by capped queen cells at inspection. A pre-swarm alert is a true positive if it precedes the swarm, giving precision P = TP/(TP+FP) and recall R = TP/(TP+FN) per swarm, with the lead time reported; the departure rule is also tested with controlled removal of known masses (0.5–2 kg). Event wakes and false alarms are counted per hive-day. Thresholds are tuned on the first weeks and frozen before the evaluation period. The packet delivery ratio (PDR) is computed from sequence numbers, and the hub logs its battery voltage to check the solar energy balance.

TABLE II.  PROPOSED ACCEPTANCE TARGETS (FROZEN BEFORE THE FIELD EVALUATION)

| Area | Metric | Target |
| --- | --- | --- |
| Weighing | Error vs. reference masses; daily drift (control) | ≤ ±20 g; reported |
| Wake-up | Departures and bursts missed; false wakes | None; ≤ 20 per hive-day |
| Swarm alert | Precision / recall of alerts before departure; lead time | > 90% / > 90%; reported |
| False alarms | Alerts without a matching event | ≤ 1 per hive-week |
| Wireless | Packet delivery ratio, hive to hub (ESP-NOW) | ≥ 95% |
| Power / reliability | Projected node battery life; unattended run | ≥ 6 months; ≥ 30 days |

References

Background research: precision beekeeping, swarming and hive signals

[1]	A. Zacepins, V. Brusbardis, J. Meitalovs, and E. Stalidzans, "Challenges in the development of precision beekeeping," Biosyst. Eng., vol. 130, pp. 60–71, Feb. 2015, doi: 10.1016/j.biosystemseng.2014.12.001.

[2]	S. Ferrari, M. Silva, M. Guarino, and D. Berckmans, "Monitoring of swarming sounds in bee hives for early detection of the swarming period," Comput. Electron. Agric., vol. 64, no. 1, pp. 72–77, 2008, doi: 10.1016/j.compag.2008.05.010.

[3]	M.-T. Ramsey, M. Bencsik, M. I. Newton, M. Reyes, M. Pioz, D. Crauser, N. Simon Delso, and Y. Le Conte, "The prediction of swarming in honeybee colonies using vibrational spectra," Sci. Rep., vol. 10, Art. no. 9798, Jun. 2020, doi: 10.1038/s41598-020-66115-5.

[4]	W. G. Meikle and N. Holst, "Application of continuous monitoring of honeybee colonies," Apidologie, vol. 46, no. 1, pp. 10–22, 2015, doi: 10.1007/s13592-014-0298-x.

Existing hive-monitoring systems

[5]	S. Cecchi, S. Spinsante, A. Terenzi, and S. Orcioni, "A smart sensor-based measurement system for advanced bee hive monitoring," Sensors, vol. 20, no. 9, Art. no. 2726, May 2020, doi: 10.3390/s20092726.

[6]	BeeHero, "Technology." [Online]. Available: https://www.beehero.io/technology

Design data: signal levels, communication and components

[7]	M. Bencsik, Y. Le Conte, M. Reyes, M. Pioz, D. Whittaker, D. Crauser, N. Simon Delso, and M. I. Newton, "Honeybee colony vibrational measurements to highlight the brood cycle," PLoS ONE, vol. 10, no. 11, Art. no. e0141926, Nov. 2015, doi: 10.1371/journal.pone.0141926.

[8]	Espressif Systems, "ESP-NOW," in ESP-IDF Programming Guide (ESP32-C3), v5.3.2. [Online]. Available: https://docs.espressif.com/projects/esp-idf/en/v5.3.2/esp32c3/api-reference/network/esp_now.html

[9]	Avia Semiconductor, "HX711: 24-bit analog-to-digital converter (ADC) for weigh scales," datasheet.

[10]	Espressif Systems, "ESP32-C3 series datasheet." [Online]. Available: https://www.espressif.com/en/support/documents/technical-documents

הערכה לשיחת סקר תכנון ראשוני (PDR)

מספר הפרויקט: P-2027-165, שם הפרויקט: BeeSense — IoT-Based Acoustic and Weight Monitoring of Honeybee Colonies

שם המנחה: ד"ר דן וילנצ'יק

שם הסטודנט: דרור בן סימון          ת.ז.: 318187747

| % | קריטריון | 1 - חלש | 2 - בינוני | 3 - מצוין |
| --- | --- | --- | --- | --- |
| 20 | הגדרת המטרה - האם מטרת הפרויקט ברורה? האם הוגדרו קריטריונים ברורים ומדידים כמותית לבחינת הצלחת הפרויקט? |  |  |  |
| 20 | הבנת הבעיה - האם ניכר שהסטודנטים מבינים את הבעיה ואת הפתרונות הקיימים? האם נערך סקר ספרות רלוונטי? |  |  |  |
| 20 | הצעת הפתרון - האם הגישה הכללית ורכיבי הפתרון הוצגו והוסברו בצורה ברורה? האם הובנו מהות העשייה, הצורך וסביבת היישום? |  |  |  |
| 20 | בדיקת היתכנות - האם הסטודנטים ביצעו מחקר בסיסי לבדיקת היתכנות פתרון לבעיה? כמו כתיבת קוד, ניתוח מעגל, ניתוח מודל מתמטי... |  |  |  |
| 20 | איכות כתיבת ה PDR - תחביר, הגדרת המטרה, סקר ספרות, תקצירים, מפרט טכני (או הצעת מחקר) והצעת תכנון מפורטים. |  |  |  |

הערות: ____________________________________________

הערכה לשיחת סקר תכנון ראשוני (PDR)

מספר הפרויקט: P-2027-165, שם הפרויקט: BeeSense — IoT-Based Acoustic and Weight Monitoring of Honeybee Colonies

שם המנחה: ד"ר דן וילנצ'יק

שם הסטודנט: דן קופר          ת.ז.: 319011102

| % | קריטריון | 1 - חלש | 2 - בינוני | 3 - מצוין |
| --- | --- | --- | --- | --- |
| 20 | הגדרת המטרה - האם מטרת הפרויקט ברורה? האם הוגדרו קריטריונים ברורים ומדידים כמותית לבחינת הצלחת הפרויקט? |  |  |  |
| 20 | הבנת הבעיה - האם ניכר שהסטודנטים מבינים את הבעיה ואת הפתרונות הקיימים? האם נערך סקר ספרות רלוונטי? |  |  |  |
| 20 | הצעת הפתרון - האם הגישה הכללית ורכיבי הפתרון הוצגו והוסברו בצורה ברורה? האם הובנו מהות העשייה, הצורך וסביבת היישום? |  |  |  |
| 20 | בדיקת היתכנות - האם הסטודנטים ביצעו מחקר בסיסי לבדיקת היתכנות פתרון לבעיה? כמו כתיבת קוד, ניתוח מעגל, ניתוח מודל מתמטי... |  |  |  |
| 20 | איכות כתיבת ה PDR - תחביר, הגדרת המטרה, סקר ספרות, תקצירים, מפרט טכני (או הצעת מחקר) והצעת תכנון מפורטים. |  |  |  |

הערות: ____________________________________________